BASI Model: 908U (Sloped Top)

How to order: Model + Heater Code with color code + Options (example 908U02000NW-1655-T1)

| Length In. (mm) | Density |  | Watts | Heater Code |  |  |  |  |  |  | Watts at 240/208V | Heater Code |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | W/Ft | W/m |  |  | 208V | Amps |  | 277V | Amps |  |  | 240/208V | Amps |
| 27.2 (690) | 375 | 1250 | 800 |  | 00800C‡-690 | 3.8 |  | 00800N‡-690 | 2.9 |  | 800/600 | 00800V‡-690 | 3.3/2.9 |
|  | 500 | 1640 | 1000 |  | 01000C‡ | 4.8 |  | 01000N‡ | 3.6 |  | 1000/750 | 01000V‡ | 4.2/3.6 |
| 37.2 (944) | 375 | 1250 | 1000 |  | 01000C‡-944 | 4.8 |  | 01000N‡-944 | 3.6 |  | 1000/750 | 01000V‡-944 | 4.2/3.6 |
|  | 500 | 1640 | 1500 |  | 01500C‡ | 7.2 |  | 01500N‡ | 5.4 |  | 1500/1125 | 01500V‡ | 6.3/5.4 |
| 47.2 (1198) | 375 | 1250 | 1500 |  | 01500C‡-1198 | 7.2 |  | 01500N‡-1198 | 5.4 |  | 1500/1125 | 01500V‡-1198 | 6.3/5.4 |
|  | 500 | 1640 | 2000 |  | 02000C‡ | 9.6 |  | 02000N‡ | 7.2 |  | 2000/1500 | 02000V‡ | 8.3/7.2 |
| 57.2 (1452) | 375 | 1250 | 1750 |  | 01750C‡-1452 | 8.4 |  | 01750N‡-1452 | 6.3 |  | 1750/1313 | 01750V‡-1452 | 7.3/6.3 |
|  | 500 | 1640 | 2500 |  | 02500C‡ | 12.0 |  | 02500N‡ | 9.0 |  | 2500/1875 | 02500V‡ | 10.4/9.0 |
| 65.2 (1655) | 375 | 1250 | 2000 |  | 02000C‡-1655 | 9.6 |  | 02000N‡-1655 | 7.2 |  | 2000/1500 | 02000V‡-1655 | 8.3/7.2 |
|  | 500 | 1640 | 3000 |  | 03000C‡ | 14.4 |  | 03000N‡ | 10.8 |  | 3000/2250 | 03000V‡ | 12.5/10.8 |
| 75.2 (1909) | 375 | 1250 | 2500 |  | 02500C‡-1909 | 12.0 |  | 02500N‡-1909 | 9.0 |  | 2500/1875 | 02500V‡-1909 | 10.4/9.0 |
|  | 500 | 1640 | 3500 |  | 03500C‡(1) | 16.8 |  | 03500N‡ | 12.6 |  | 3500/2625 | 03500V‡ | 14.6/12.6 |
| 83.2 (2112) | 375 | 1250 | 2750 |  | 02750C‡-2112 | 13.2 |  | 02750N‡-2112 | 9.9 |  | 2750/2066 | 02750V‡-2112 | 11.5/9.9 |
|  | 500 | 1640 | 4000 |  | 04000C‡(1) | 19.2 |  | 04000N‡ | 14.4 |  | 4000/3000 | 04000V‡(1) | 16.7/14.4 |

Other single-phase voltages available, consult factory. All models listed are single-phase.

‡Standard colors - Add W for white finish or A for almond finish to the end of catalog number when ‡ is indicated.

(1) If control option R7 is required for these heaters, (Qty 2) R7 Triac proportional relays are required.

Factory Built-in Options

| Code | Description |  |
| --- | --- | --- |
| D1 | Disconnect Switch, Three-pole, 600V, 40 Amp |  |
| F | Front Air Outlet (BAI Only) |  |
| R1 | 24V Relay, 240V, 22 Amp; 277V, 19 Amp |  |
| R2 | 24V Relay with Transformer, 240V, 22 Amp; 277V, 19 Amp |  |
| R3 | 24V Double Pole Relay, 240V, 25 Amp; 480V, 12.5 Amp |  |
| R4 | 240V Relay, Single-pole, 240V, 25 Amp; 480V, 12.5 Amp |  |
| R5 | 240V Relay, Double-pole, 240V, 25 Amp; 480V, 12.5 Amp |  |
| R6 | Pneumatic Relay, 480V, 25 Amp |  |
| R7 | 24V Proportional Relay |  |
| S | Smooth Finish on Heater Back |  |
| T1 | Tamperproof Thermostat, Single-pole, 240V, 30 Amp; 480V, 15 Amp |  |
| T2 | Tamperproof Thermostat, Double-pole, 480V, 15 Amp |  |
| X1 | 240V Transformer – 50 VA |  |
| X2 | 24V Transformer – 50 VA |  |

Field-Installed Options

(shipped loose for field installation into heater)

| Part No. | Description |  |  |
| --- | --- | --- | --- |
| 908-124609¶ | BASI Inside corner |  |  |
| 908-218356¶ | BASI Outside corner |  |  |
| 909-124610¶ | Square Pedestal Kit | 3-4” High |  |
| 908-124611¶ | Sub Base Heater Length In. (mm) | 27.2 (690) |  |
| 908-124612¶ |  | 37.2 (944) |  |
| 908-124613¶ |  | 47.2 (1198) |  |
| 908-124614¶ |  | 57.2 (1452) |  |
| 908-124615¶ |  | 65.2 (1655) |  |
| 908-124616¶ |  | 75.2 (1909) |  |
| 908-124617¶ |  | 83.2 (2112) |  |
| 908-218276¶ | Empty Cabinet Length In. (mm) | 27.2 (690) |  |
| 908-218277 ¶ |  | 37.2 (944) |  |
| 908-218278 ¶ |  | 47.2 (1198) |  |
| 908-218279 ¶ |  | 57.2 (1452) |  |
| 908-218280 ¶ |  | 65.2 (1655) |  |
| 908-218281¶ |  | 75.2 (1909) |  |
| 908-218282 ¶ |  | 83.2 (2112) |  |
| 908-124618¶ | Recessed Trim Kit Heater Length In. (mm) (BAI Only, Option F Required) | 27.2 (690) |  |
| 908-124619¶ |  | 37.2 (944) |  |
| 908-124620¶ |  | 47.2 (1198) |  |
| 908-124621¶ |  | 57.2 (1452) |  |
| 908-124622¶ |  | 65.2 (1655) |  |
| 908-124623¶ |  | 75.2 (1909) |  |
| 908-124624¶ |  | 83.2 (2112) |  |
| 908-124628¶ | Control Box – 12” | 12 (305) |  |
| 909-124629 | 24V Relay Kit, Single-pole, 240V, 22 Amp; 277V, 19 Amp |  |  |
| 909-124630 | 24V Relay Kit, Double-pole, 240V, 25 Amp; 480V, 12.5 Amp |  |  |
| 909-124633 | 24V Relay Kit with Transformer, 208V, 22 Amp |  |  |
| 909-124634 | 24V Relay Kit with Transformer, 240V, 22 Amp |  |  |
| 909-124635 | 24V Relay Kit with Transformer, 277V, 19 Amp |  |  |
| 909-224759 | Tamperproof Single-pole Thermostat Kit |  |  |
| 909-224760 | Tamperproof Double-pole Thermostat Kit |  |  |

¶ Standard colors - Add W for white finish or A for almond finish to the end of part number.

Front View

Rear View

Left Side View

Bottom View

Architect’s and Engineers Specification

Heating Elements – Stainless steel elements with aluminum fins float on high temperature nylon bushings for quiet operation.

Housing – Cabinet is constructed of 20-gauge steel with a 16-gauge steel front cover. The unit has a polyester powder paint finish in white or almond.

Inlet/Outlet Grilles – The fresh air inlet grille is located on the front of the unit. An extruded aluminum outlet grille discourages tampering with the heating element.

Built-in Controls – Linear limit, automatic reset thermal cutout.

Installation Requirements – Field or factory-installed low voltage relay and thermostat kits. These units are not for residential use in the United States.